馬偕紀念醫院臨床技能訓練學習單

| 訓練技能 | 氣管插管 (Endotracheal Intubation) |  |
| --- | --- | --- |
| 學習目標(Learning Objectives） | - 評估呼吸道 - 維持呼吸道 - 知道插管的時機 | - 操作插管前的準備 - 正確插管 - 確認氣管插管 |
| 學習重點 | 適應症 - Respiratory arrest / failure - Hypoventilation/Hypercarbia - PaCO2 >55 mmHg - Arterial Hypoxemia refractory to oxygen - PaO2 <55mmHg under room air, PaO2<70mmHg on 100% face mask - Respiratory Acidosis - Airway obstruction - Glasgow Coma Scale <=8 - Need for prolonged Ventilator support - Class III or IV hemorrhage with poor perfusion - Severe flail chest or pulmonary contusion - Multiple trauma, Head Injury and abnormal mental status - Inhalation Injury with edema at cords - Protection from aspiration Difficult Airway的評估 - Assess cervical spine mobility. - Assess mouth opening (three fingers between the incisors) - Assess oral access (Mallampati scale). - Assess laryngoscopic geometry (thyromental distance ≤6 cm ). - Evaluate for obstruction Mallampati scale： Grade 1: pillars, soft palate and uvula visible Grade 2: pillars, soft palate visible, but uvula masked by the base of the tongue Grade 3: soft palate only visible Grade 4: soft palate not visible. Grade 1~2 : 容易插管 Grade 3~4 : 困難插管 | 設備 Suction, Oxygen, Airway (laryngoscope, ET tubes, stylet), Monitoring & Equipment ET tubes size (內徑 ID) 成人：Male 7.0, 7.5 mm Female 7.0, 6.5mm 深度：約22公分（嘴角） Cuff: 注射3~5 ml Air, cuff pressure 約 15~25 cmH2O 保持氣道暢通 人工氣道可用作控制舌頭免其下墮，以保持氣道暢通。(oral airway or nasal airway) Mask Ventilation : 維持呼吸道的暢通,以面罩Mask有效供應氧氣,是airway management 最重要的第一步. 如果在仰頭Neck Extension以及抬下巴Chin Lift 之後,仍然無法順利換氣,應立即考慮置入口咽人工氣道或鼻咽人工氣道 Laryngoscopy: 喉頭鏡(laryngoscope)的葉片(blade),大部分情況是使用Macintosh彎的葉片,葉片正確的位置要放到舌根(tongue base)與會厭軟骨(epiglottis)之間,這個轉折的凹陷稱做vallecula. 此外,一般來說在頭部墊個10公分高的枕頭, airway的Axis軸會比較直,有利於插管. 插管的困難度 決定於在 laryngoscopy 下 Epiglottis 以及Vocal Cord 所能看到的程度,分成 Grade 1, 2, 3, 4, Grade 4 最困難插管 插管後確認 - 聽診 (Chest & Stomach都要聽) - End-tidal CO2 monitor 臨床技能中心製作 2008/4/3 麻醉科 徐永偉醫師審核 |